Core Flight Computer Requirements

Sub-systems

- Microcontroller

  - Firmware

  - Bus channels

- Power management

  - Battery

  - Voltage Regulation

- Sensors

  - GPS

  - Flight data

- Outputs

  - Storage

  - Pyro charges

  - indicators

Construction

- Core: 3-stack 2.30 diameter (3 in body tube minimum)

  - 1. Telemetry, GPS, sensors?

  - 2. Microcontroller, barometer, IMU

  - 3. Power, regulation,

Other

- Meeting song playlist

- Flight computer names (form, vote)

- Github organization

- Notes are in teams

Power management

Battery Selection

- Lithium-ion (Li- Ion) or Lithium-Polymere (LiPo)

- Capacity: C [Amp-Hours]

- Cell number: parallel = +C, series = +V

- C-rating: current the battery can be charged/discharged at (1C with .5 Ah = 0.5 A)

- Connector (XT60 recommended)

Voltage Regulation

- Different kinds of regulators

- Switching regulator IC: buck converter most likely

  - Includes passive components on outside of IC (careful of capacitor ESR and RMS)

    - ICs have specific components that require using the right values.

  - Conserves power by stepping down voltage and stepping up current

Microcontroller system

Teensy 4.1 – Breakout board

Pro: Very easy to implement

- Has onboard SD card and has a very fast processing speed ~ 600 MHz

Con: Hard to implement in stack design, has rigid structure

ESP32 – Microcontroller module

Pro: Easy to integrate

- Already implements Wifi and Bluetooth (has its own antenna?), excels in low power (sleep) states, fast speed

Con: Still larger but great option

STM32 – Microcontroller Unit

Pro: Super tailorable

- Lots of resources, firmware opportunities, industry standard

Con: Can be very complex, we have to design the outer circuits like communication

Sensor Integration

We will need (core flight computer)

- Gyroscope: Measures 3-axis orientation. Part of IMU.

  - Drift prone

- Accelerometer: Measures acceleration to determine 3-axis orientation. Part of IMU.

  - Vibration prone

- Barometer: Measures air pressure and temp to determine altitude.

  - Can be influenced by weather inconsistency.

- Voltmeter: Measures the battery capacity.

- GNSS Receiver (GPS): Determines position using satellites

- Transceiver: Communicates with computer for real time data communication. Requires more electronics.

- RF transceiver circuit: Send flight-data to ground

This means lots of analog to digital conversion (ADC), and 2 antennas. (unless we didn’t want live telemetry).

3D modeling opportunity with orientation

Each sensor has its own parameters which often include

- Operating range ( where can it measure)

- Resolution

- Accuracy (TCO, zero-offset, noise density)

Communication between sensor and uC: Typically  I2C, SPI, and UART

- Goal is to make the system modular/expandable, # of channels on uC is important

Outputs

On-Board flight data storage

- SD card, flash, both

Indicators

- LEDs (RGB gives many options) for visual indication

- Buzzer/speaker for auditory indication

Pyro charges

- Use electric ignitors

- Require safety and protection circuits for high current

- Indictor may be good

Battery/Power management

Inputs: BAT+, BAT- (DC Power)

Outputs: 3.3V, GND, Pyro Supply, VBat sense

Microcontroller (STM32)

Inputs: 3.3V, GND, I2C SDA, SPI MISO, UART RX, ADC, interrupts

Outputs: I2C SCL, SPI SCK/MOSI/CS, UART TX, GPIO, PWM

IMU

Inputs: 3.3V, GND, SCL

Outputs: Interrupt (optional)

Barometer

Inputs: 3.3V, GND, SCL

Outputs: Interrupt (optional)

GPS

Inputs: 3.3V, GND, UART RX, antenna RF

Outputs: UART TX

Telemetry

Inputs: 3.3V, GND, SPI MOSI/SCK/CS, RESET antenna RF

Outputs: SPI MISO, IRQ, BUSY

Data Storage

Inputs: 3.3V, GND, SPI MOSI/SCK/CS

Outputs: SPI MISO

Pyro Channels

Inputs: Pyro supply, return, GPIO commands, arming input

Outputs: Deployment outputs, continuity feedback, arm status

Indicators

Input: 3.3 V, GND, GPIO/PWM

Outputs: Light, Sound